Packet Tracer - Troubleshooting a VLAN Implementation Scenario 1

Topology

Addressing Table

| Device | Interface | IPv4 Address | Subnet Mask | Switch Port | VLAN |
| --- | --- | --- | --- | --- | --- |
| PC1 | NIC | 172.17.10.21 | 255.255.255.0 | S1 F0/11 | 10 |
| PC2 | NIC | 172.17.20.22 | 255.255.255.0 | S1 F0/18 | 20 |
| PC3 | NIC | 172.17.30.23 | 255.255.255.0 | S1 F0/6 | 30 |
| PC4 | NIC | 172.17.10.24 | 255.255.255.0 | S2 F0/11 | 10 |
| PC5 | NIC | 172.17.20.25 | 255.255.255.0 | S2 F0/18 | 20 |
| PC6 | NIC | 172.17.30.26 | 255.255.255.0 | S2 F0/6 | 30 |

- Objectives

Part 1: Test Connectivity between PCs on the Same VLAN

Part 2: Investigate Connectivity Problems by Gathering Data

Part 3: Implement the Solution and Test Connectivity

Scenario

In this activity, you will troubleshoot connectivity problems between PCs on the same VLAN. The activity is complete when PCs on the same VLAN can ping each other. Any solution you implement must conform to the Addressing Table.

Test Connectivity between PCs on the Same VLAN

From the command prompt on each PC, ping between PCs on the same VLAN.

Can PC1 ping PC4? no

Can PC2 ping PC5? no

Can PC3 ping PC6? no

Investigate Connectivity Problems by Gathering Data

Verify configuration on the PCs.

Verify if the following configurations for each PC is correct.

IP address

Subnet mask

Verify the configuration on the switches.

Verify if the following configurations on the switches are correct.

Ports assigned to the correct VLANs.

Ports configured for the correct mode.

Ports connected to the correct devices.

Document the problem and the solutions.

List the problems and the solutions that will allow these PCs to ping each other. Keep in mind that there could be more than one problem or more than one solution.

PC1 to PC4

Explain the connectivity issues between PC1 and PC4.

- PC1 is on VLAN 30 instead of VLAN 10, and the port for G0/1 on S1 is configured as an access.

Record the necessary actions to correct the issues.

- switchport mode trunk command on G0/1 ints for S1 and S2.

- switchport access vlan 10 command on F0/11 on S2

PC2 to PC5

Explain the connectivity issues between PC2 and PC5.

- Pc5 is connected to the wrong port and F0/18 is assigned the wrong VLAN.

Record the necessary actions to correct the issues.

- Switch PC5 from port F0/17 to F0/18 on S3 and assign F0/18 to VLAN 20.

- switchport mode trunk command on G0/1 ints for S1 and S2

PC3 to PC6

What are the reasons why connectivity failed between the PCs?

- The IP address for PC6 is not correct, G0/1 on S1 is configured in access mode, and F0/6 on S3 is not assigned to a VLAN.

Record the necessary actions to correct the issues.

- Change the IP address for PC6 to 172.17.30.26.

- Assign F0/6 on S3 to VLAN 30.

- Switchport mode trunk command on G0/1 ints for S1 and S2.

Implement the Solution and Test Connectivity

Verify PCs on the same VLAN can now ping each other. If not, continue to troubleshoot.

Suggested Scoring Rubric

Packet Tracer scores 70 points. Documentation in Part 2, Step 3 is worth 30 points.