Problem no.1 – A prestressed concrete beam has a width of 300mm and an overall depth of 600mm. The prestressing tendons are placed at a distance “e” below the neutral axis of the beam and the applied prestressing force is P in kN. There is 15% loss of prestress.

- Determine the value of P and e when the compressive stress in the entire cross-section is 21 MPa.

The compressive stress at the top and bottom of the beam is given by the formula:

fc=-Pbh±6Pebh2

For the stress in the entire cross-section to be uniform, P must be applied at the center, hence e=0 and:

P=fc+bh	P=21300600

P=3780kN

- Determine the value of P and e when the compressive stresses at the top and bottom of the beam are 2 MPa and 12 MPa, respectively.

fc top=-Pbh+6Pebh2	-2=-P300(600)+6Pe300(600)2

P=36,000,000100-e

fc bot=-Pbh-6Pebh2	-12=-P300600-6Pe300(600)2

P=216,000,000e+100

P=P		36,000,000100-e=216,000,000e+100

e=5007=71.43 mm

P=216,000,00071.4286+100

P=1,260,000 N

P=1,260 kN

- Determine the values of P and e when the compressive stresses at the top and bottom of the beam are 0 MPa and 16 MP, respectively.

Since the stress at the top s zero, P acts at h/3 from the bottom of the beam, or e=h/2 – h/3 = h/6

e=6006=100mm

P=12fcx h x b	P=1216600(300)

P=1440000 N

P=1440 kN

Problem no. 2 – The section of a prestressed double-tee concrete floor joist is shown in the Figure. The prestressing force in each tee is 750 kN. Unit weight for concrete is 23 kN/m3.

The properties of the double-tee section are:

Area=220,000 mm2	y2= 270 mm

l= 1890 x 106 mm4	y3= 75 mm

y1= 90 mm		Simple span, L=8 m

Service loads on floor:	Dead load = 2.5 kPa

Live load =6 kPa

- Determine the initial stress at the bottom fibers due to prestressing force along.

Loads:

wd=pd x b=2.52.5=6.25kNm

wl=pl x b=62.5=15kNm

wb=γcA=23.5 x (220,00010002)=5.17kNm

Moment at midspan, M=wL28=26.42(8)28=211.36 kN-m

Stresses due to initial presses:

e=y2-y3=270-75=195 mm

ftop=-PA+Pecl	ftop=-2(750,000)220,0000+(220,000 x 195)(90)1890x106

ftop=7.11 MPa

fbot=-PA-Pecl	fbot=-2750,000220,0000-(220,000 x 195)(270)1890x106

fbot=-48.604 MPa

- Determine the stress at the bottom fibers due to service load and prestressing force. Assume that there is a loss of prestress of 20% at service loads.

- Calculate the additional load can the floor carry so that the stress at the bottom fibers at the midspan is zero.