ZTE MF100 UNLOCK

Download:
1.QPST - http://www.mediafire.com/download.php?i8l66me6v9z63ot
2.MF100 Firmware - http://dc-files.com/files/zte/modems/MF100/firmware_update/DL_MF100_ETS_EG_EUV1.00.00.zip
3.DCCRAP - http://www.mediafire.com/?gna1mmwdgym
4.ZTE Driver - http://dc-files.com/files/zte/modems/MF100/driver/ZTEDrvSetup.zip
5.GlobeVisibility - http://www.mediafire.com/?4b178go5smtaf4m

Instructions:
1. extract all zip or rar files.
2.a. install QPST.exe.
2.b. install ZTEDrvSetup.exe. - skip if dongle not bricked/dead.
3. run "QPST Configuration">click "Add New Ports">uncheck "Show Serial and USB/QC Diagnostic ports only"> add all ports "COMX -USB/unknown" - X is a number.
4. click the added port and goto "Start Clients">EFS EXPLORER> then ok ok.
5. delete all files except folders in the EFS EXPLORER.
6. drag the files inside this folder. "\DL_MF100_ETS_EG_EUV1.00.00\DL_MF100_ETS_EG_EUV1\ ETS_EG_P671A1V1.0.0B06\3.Download_File\" and wait until finished. - from the files you extracted
7. run MF100UpdateToolV1.9.exe from "\DL_MF100_ETS_EG_EUV1.00.00\DL_MF100_ETS_EG_EUV1\ ". - from the files you extracted
8. in MF100UpdateToolV1.9.exe check "Special Item" and click "Software Version" look for "\DL_MF100_ETS_EG_EUV1.00.00\DL_MF100_ETS_EG_EUV1\ ETS_EG_P671A1V1.0.0B06\3.Download_File\" from the files you extracted.
9. click the "purple arrow" pointing to the right and wait until finished.[theres a time that it will search for the file ".sdi" just click close/cancel]
10. run DCCRAP, Select manufacturer "ZTE datacards",click "Detect Card" or the big magnifying glass, goto Flash and click "Write Dashboard" and use the GlobeVisibility.ISO you downloaded earlier - wait until finish.
11. enjoy your unlock SmartBro dongle. 
12. go outside and buy halo-halo. 

In any steps if you have trouble try close the finish program you use or reset/unplug and plug dongle.
Use any of this program or TUT at your own risk.
If you can't understand the TUT so sorry for you.
Tested in my Dead/no Autorun 16digit code MF100

credit to  TIMMY for the theory, kiLLi-kiLLerZ for the DCCRAP flash writing and ExcL for the tut of changing the dashboard and all of Symb family. <just click thier names for the reference>

================================================== =
Requested:

Anyone who need the pictures just do it step by step.. warning: use at your own risk!
in every step close the program that has been used except what your see in the picture..
If the color is RED it must be it.
If the color is YELLOW it can be any number or letter etc.

kasi hindi na dead yung MF100 ko iba po ung iba na makikita nyo sa picture..

To be continue…..