ZTE M190 UNLOCK

Download nyo muna ang mga kailangan sa pag openLine. (Mag effort kayo..)  


Nka attach na jan ang notepad kung saan pwede e download..

1. Install QPST, then Open QPST Configuration. (nakaplug na ang stick by this time) Go to PORTS Click "Add New Port" and select the port of your USB Modem (See Attachment). "EX: port:COM3, port label:COM3" (See attachment kung san makita ang PORT ng stick), then click OK. You should see there SURFXXXX under Phone column. Click "Start Client (nasa taas na menu bar)", then click "EFS Explorer". You will see there the port with SURFXXXX na label, click OK. You will be presented with folders and files. Do not touch the folders. Just to be sure, save a copy of the following files before you delete them, 9 files all in all (this will serve as your original SmartBro MF190 Files). You can save a copy by highlighting all the files then right click. After saving a backup, DELETE the listed files.
1. config
2. reset_cntr.bin
3. pbm_phone_uid.dat
4. pbm_master_v_20.dat
5. ZTEMODEM.ISO
6. UimEfsAPDULog.Txt
7. NODOWNLOAD.FLG
8. SWITCH.TMP
9. AUTORUN.FLG
After deleting the files, close "EFS Explorer". You will be prompted to reset the device, just choose "No". Close "QPST Configuration". 

2. Update firmware using "dl_mf190s_eti_eg_euv1.00.00" firmware file(approx.15mins.). After the update, the device will reset automatically and will install the dashboard na "ETISALAT"(I do not know if ano ang mangyayari if you will interrupt the installation process. Basta ako, I let it install). Base on my experience, it will ask for the drivers. Ang ginawa ko, I direct the path to SMARTBro Folder under Program Files, then sa Drivers folder, then depende na sa OS nyu.There are times when ma-pause and installation because hindi pa nainstall ang drivers. Once installed na ang drivers, the installation process will continue. Ang nangyari sakin, 2 Dashboards will showup, isa sa SmartBro and sa Etisalat. Just let it pop up then close/exit.

3. Unplug then replug the USB stick, Next is to update the firmware this time using"ZTE MF190 Movistar ES" firmware. BUT BEFORE that, repeat Step 1 wherein you need to open QPST Configuration again. Do not touch the folders, you can save a backup copy of the files before deleting. Still DELETE the listed files.
1. config
2. reset_cntr.bin
3. pbm_phone_uid.dat
4. pbm_master_v_20.dat
5. ZTEMODEM.ISO
6. UimEfsAPDULog.Txt
7. NODOWNLOAD.FLG
8. SWITCH.TMP
9. AUTORUN.FLG
After deleting the files, close "EFS Explorer". You will be prompted to reset the device, just choose "No". Close "QPST Configuration". 

4. This time update firmware using “ZTE MF190 Movistar ES” file (20-25mins.). Double click the file, wait for it to detect the device then proceed. After ma update, the device will reset automatically and will install the dashboard na "Movistar" and will ask for the drivers again. Still follow the same process. Now, 3 dashboards will pop-up, 1 SmartBro, 1 Etisalat, and 1 Movistar. Let the dashboards pop-up then simply close/exit.

5. Back to Step 1 again, open QPST Config, then save a backup copy of the files except the folders if you want. I DELETED ALL the files EXCEPT the FOLDERS, and drag & drop the files from the "original SmartBro MF190 folder", all 9 files drop to where I deleted the listed files:
1. config
2. reset_cntr.bin
3. pbm_phone_uid.dat
4. pbm_master_v_20.dat
5. ZTEMODEM.ISO > Wag na e sali, Use this Dashboard para perfect ang Stick mo.
http://www.4shared.com/file/A0-oJcbZ/ZTEMODEM.html?

6. UimEfsAPDULog.Txt
7. NODOWNLOAD.FLG
8. SWITCH.TMP
9. AUTORUN.FLG
After deleting and replacing the files, close "EFS Explorer". You will be prompted to reset the device, just choose "No". Close "QPST Configuration". 

6. This time, unplug the USB stick, uninstall the 3 dashboards yung smartbro na dashboard, etisalat,and movistar na dashboard, then clean / fixed registry using ccleaner na software. You can restart your computer after uninstalling all 3 softwares to prepare for fresh install once ireplug natin ang USB Stick.

7.Ikabit ang USB Stick na MAY IBANG SIM maliban sa smartbro. Then mag-ooautorun sya para mainstall ang dashboard na "Mobily Connect Card" at corresponding Drivers. You will be amazed na accepted ang sim and merong signal. OPENLINE na SmartBro ZTE MF190 Stick mo!!

REQUIREMENTS

- http://www.mediafire.com/?ljaojkg2f1syq3t  - QPST_Setup.rar

- http://www.mediafire.com/?bmhwrq2hzo67gv3  -  dl_mf190s_eti_eg_euv1.00.00.rar

- http://www.mediafire.com/?m62t8dv0w7v4sdb  -  DL_MF190_MOVISTAR_ES_EUV1.00.00.zip

- http://www.4shared.com/rar/MRzwR5cL/ORIGINAL_MF190_BACKUP__genxy_.html?  -  ORIGINAL_MF190_BACKUP

- http://www.4shared.com/file/A0-oJcbZ/ZTEMODEM.html?  -  ztemodem.iso

FROM: http://www.phcorner.net/threads/share-openline-smartbro-mf190-100-tested-by-me.55373/

in cause failure

PROCEDURE

1st. DL muna ito ZTE DASHBOARD/DRIVER (http://smart.com.ph/smartbro/shared/resources/Power_Plugit-Win-MF180.zip)..

2nd. Install it..
install nyu yan na hndi nakasaksak ang modem .

after ma install, salpak nyu na ang modem (WITHOUT SIM) sa mismong likod ng pc nyu.

3rd. Download QPST_TOOL (http://www.mediafire.com/?lqsva0zlwi9t1pt)
after ma DL, install nyu na agad..

THEN, Download your corresponding files.
MF180 ORIGINAL BACKUP (http://www.mediafire.com/?57hphazc6tww746)
MF190 ORIGINAL BACKUP (http://www.mediafire.com/?wnwsvz7w52dpt1b)
MF100 ORIGINAL BACKUP (http://www.mediafire.com/?0th241qtdxege8s)

Then pagkatapos ma DL, extract it to a specified folder..

Ayan kompleto na rekados. pwede na siguro to simulan..
Siguraduhin na nakasaksak at nainstall na ng maigi ang modem natin.

1st step open ang QPST TOOL..
http://img411.imageshack.us/img411/1766/19965891.jpg

2nd. Sundan lang ang nasa pic
http://img545.imageshack.us/img545/5849/65301965.jpg

3. Sundan parin ang nasa pic
http://img406.imageshack.us/img406/812/35086816.jpg

4. ok lang.
http://img403.imageshack.us/img403/5459/38252409.jpg

5. delete nyo yang mga files except sa mga folders.
except sa mga folders.
ganito.
http://img85.imageshack.us/img85/4232/54456812.jpg

6. punta kayo sa folder kung saan ninyo na extract ang orig files ng mf180 or mf190.

at idrag nyo lang ito lahat sa QPST tool.
http://img827.imageshack.us/img827/5711/47498205.jpg

medyo may katagalan yan, kasi cdfs yan.
hintay hintay lang..

after few minutes pagkatapos ma copy lahat,
exit nyo na ang QPST.
then click ok pag may magpa promt.
then restart your pc..

AFTER ng ginawa natin.
eto na sya..

http://img703.imageshack.us/img703/5161/68652068.jpg